Today

You said

I have 3 related embedded projects written using STM32CubdeIDE for a single product: a boot-only first-stage bootloader, a second-stage bootloader, and an application. How can I use the debug capabilities of STM32CubeIDE with a J-Link MINIE to walk through the code and follow the handoff from one project to the next?

Copilot said

You can absolutely step through a multi-stage boot flow on an STM32F401 with a J-Link Mini, but you have to treat the three images as separate debug sessions and make STM32CubeIDE “attach” to the MCU at the right moment rather than always resetting and loading code. The core idea is:

Use normal debugging for the first-stage bootloader, then switch to “Attach to Running Target” for the second-stage bootloader and application once execution has jumped to them.

That lets you follow the handoff without disturbing the CPU state, stack pointer, or vector table that each stage sets up.

🧭 What you’re trying to achieve

You want to:

- Start debugging in the first-stage bootloader (FSBL).

- Let it load the second-stage bootloader (SSBL) into flash or RAM.

- Watch the FSBL jump to the SSBL’s reset vector.

- Then debug the SSBL as it runs.

- Let the SSBL load the application and jump to it.

- Attach again and debug the application.

This is doable, but only if each project’s debug configuration is set up to not reset the chip and not reprogram flash when you attach.

🧩 How to configure each project in STM32CubeIDE

1) First-stage bootloader (FSBL)

This is the only project you debug in the normal way.

- Debug configuration → Startup tab

  - ✔ Reset: Connect under reset

  - ✔ Load executable: Enabled

  - ✔ Set program counter at reset*: Enabled

- Start debugging normally.

- Set breakpoints where it:

  - Validates the SSBL image

  - Sets MSP and jumps to SSBL’s reset handler

When you hit the jump instruction (usually bx r0), stop. Don’t step into it yet.

2) Second-stage bootloader (SSBL)

This project must be debugged using Attach to Running Target.

- Debug configuration → Startup tab

  - ✘ Load executable: Disabled

  - ✘ Reset on connect: Disabled

  - ✘ Set program counter: Disabled

- Debugger tab

  - Interface: SWD

  - Speed: 4000–8000 kHz (J-Link Mini is fine)

- Startup scripts: leave empty

This ensures CubeIDE does not overwrite memory or reset the MCU when you attach.

3) Application

Same configuration as SSBL:

- No reset

- No flash programming

- No PC initialization

This lets you attach after the SSBL jumps into the application.

🧵 Step-by-step workflow to follow the handoff

Step 1 — Debug the FSBL normally

Start the FSBL debug session. Let it run until just before it jumps to the SSBL.

Set a breakpoint at the handoff:

c

Copy

typedef void (*pFunction)(void);

uint32_t ssbl_sp = *(__IO uint32_t*)SSBL_START_ADDR;

uint32_t ssbl_pc = *(__IO uint32_t*)(SSBL_START_ADDR + 4);

__set_MSP(ssbl_sp);

pFunction JumpToSSBL = (pFunction)ssbl_pc;

JumpToSSBL();   // <— breakpoint here

When you hit the breakpoint, do not step into the jump.

Step 2 — Run the FSBL until after the jump

Press Resume (F8) so the CPU executes the branch and is now running inside the SSBL.

The debugger will lose the symbol context (expected).

Step 3 — Attach the SSBL debugger

Now:

- Stop the CPU (pause button)

- Switch to the SSBL project

- Run Debug → Attach to Running Target

CubeIDE will load SSBL symbols only, without touching memory or resetting the chip.

You can now step through the SSBL exactly as if you had started there.

Step 4 — Repeat for the application

When the SSBL is about to jump to the application:

- Set a breakpoint at the jump

- Let it run past the jump

- Pause the CPU

- Switch to the application project

- Attach to Running Target

Now you’re debugging the application with full symbols.

🧠 Important details that make this work

1. Each image must be linked to its real runtime address

Your linker scripts must match the actual flash layout:

- FSBL at 0x08000000

- SSBL at something like 0x08004000

- Application at something like 0x08008000

If the SSBL or application are linked to 0x08000000, the debugger will misinterpret the PC and stack.

2. J-Link Mini is ideal for this

It supports:

- SWD attach without reset

- Halting the CPU after a branch

- Reading registers and memory without disturbing execution

ST-Link struggles more with attach-without-reset.

3. You cannot “step through” the actual branch between projects

Once the FSBL executes bx r0, the debugger loses symbol context. That’s why the attach-after-jump method is required.

4. If you want to see the exact moment of handoff

Set a breakpoint at the first instruction of the SSBL:

- In the SSBL project, open the .map file

- Find the reset handler address (e.g., 0x08004004)

- In the FSBL debug session, manually set a breakpoint at that address:

  - Run → Add Address Breakpoint…

Then:

- Resume FSBL

- CPU jumps to SSBL

- Breakpoint hits immediately

- Now attach SSBL symbols

This is the cleanest way to “see” the handoff.

🧪 A practical workflow you’ll probably adopt

Most embedded engineers doing multi-stage bootloaders end up using this pattern:

- Debug FSBL normally.

- Add an address breakpoint at the SSBL reset vector.

- Resume.

- When it hits, attach SSBL symbols.

- Repeat for the application.

It’s fast, reliable, and doesn’t require any special hardware tricks.

If you want, I can help you set up the linker scripts, memory map, or the exact CubeIDE debug configuration for each project so the attach flow is smooth and predictable.

You said

Run → Add Address Breakpoint… does not appear as a choice.

Copilot said

You’re right to notice that Run → Add Address Breakpoint… is not available in STM32CubeIDE. That menu item exists in some Eclipse CDT distributions, but ST removed it from CubeIDE’s UI. The good news is that you can still set an address breakpoint—you just have to use one of the supported paths that ST kept.

Below are the three reliable ways to set an address breakpoint in CubeIDE when debugging STM32 bootloaders, plus how to use each one in your multi-stage handoff workflow.

🧩 1. Use the Breakpoints View (most reliable)

This is the method that always works, regardless of project type.

Steps

- Start a debug session (FSBL in your case).

- Open the Breakpoints view: Window → Show View → Breakpoints

- In the Breakpoints view, click the blue “+” icon (Add Breakpoint).

- Choose C/C++ Address Breakpoint.

- Enter the address, e.g.:

Code

Copy

0x08004004

- Leave the rest at defaults and click OK.

Why this works well

- Works even if the symbol file doesn’t contain that address.

- Works before or after the code is loaded.

- Works even if the debugger has lost symbol context after a jump.

🧩 2. Use the Disassembly View (fastest when you’re already paused)

If the CPU is halted and you know the address, you can jump directly to it.

Steps

- While debugging, open the Disassembly view: Window → Show View → Disassembly

- In the Disassembly view, press Ctrl+L (Go to Address).

- Enter the address (e.g., 0x08004004).

- When the disassembly jumps to that location, right-click the instruction → Toggle Breakpoint.

Why this is useful

- Lets you visually confirm you’re at the correct reset handler.

- Works even if the debugger has no symbols for the SSBL or application.

🧩 3. Use the Debugger Console (J-Link GDB Server)

This is the most direct method and works even when the UI is uncooperative.

Steps

- In a debug session, open the Debugger Console tab.

- Type:

Code

Copy

break *0x08004004

- Press Enter.

Notes

- Use break or hb (hardware breakpoint) depending on your preference.

- J-Link supports multiple hardware breakpoints in flash.

🧭 How this fits into your multi-stage bootloader debugging

You can now do the clean handoff tracing you wanted:

FSBL → SSBL

- In the FSBL debug session, set an address breakpoint at the SSBL reset vector:

  - Example: 0x08004004

- Resume execution.

- When the breakpoint hits, the CPU is now executing inside the SSBL.

- Pause, switch to the SSBL project, and Attach to Running Target.

SSBL → Application

- Same pattern:

  - Set an address breakpoint at the application’s reset vector.

  - Resume.

  - When it hits, attach using the application’s debug configuration.

This gives you a clean, deterministic way to “catch” each stage immediately after the jump.

🧠 A subtle but important detail

Address breakpoints work even if the debugger has no symbols loaded for the target code. That’s why they’re perfect for multi-stage bootloaders where each image has its own vector table and its own symbol file.

🔧 Quick sanity check

To make sure the address you’re breaking on is correct:

- Open the .map file for the SSBL or application.

- Look for the symbol _start, Reset_Handler, or whatever your startup file defines.

- The reset handler is always at offset +4 from the image base.

Example:

Code

Copy

SSBL_START_ADDR = 0x08004000

Reset_Handler   = 0x08004004

If you want, I can help you verify the exact addresses from your linker scripts or map files so you can set the breakpoints with confidence.